# FWD_Advanced Embedded Diploma

Automotive Door Control System Design

By Mohamed Samy

Table Content

1. Block Diagram (Req. 1)	1

2. Static Design (Req. 2)	2

2.1. Layered Architecture (ECU1)	2

2.1.1. On Board Components:	2

2.1.2. Modules:	2

2.1.3. Folder structure:	3

2.1.4. Typedefs and Enmus used:	4

2.1.5. APIs and Private functions:	5

2.2. Layered Architecture (ECU2)	11

2.2.1. On Board Components:	11

2.2.2. Modules:	11

2.2.3. Folder structure:	12

2.2.4. Typedefs and Enmus used:	13

2.2.5. APIs and Private functions:	14

3. Dynamic Design (Req. 3)	19

3.1. On Board Components State Machine (ECU1)	19

3.2. ECU State Machine (ECU1):	20

3.3. ECU Sequence Diagram (ECU1):	21

3.4. CPU Load (ECU1):	21

3.5. On Board Components State Machine (ECU2)	22

3.6. ECU State Machine (ECU2):	23

3.7. ECU Sequence Diagram (ECU2):	24

3.8. CPU Load (ECU2):	24

4. CAN Bus Load (Req. 3)	25

# Block Diagram (Req. 1)

System block diagram that show all system blocks and design constraints

# Static Design (Req. 2)

## Layered Architecture (ECU1)

### On Board Components:

- Door Sensor (Digital)

- Speed Sensor (Analog)

- Light Switch (Digital)

### Modules:

- Timer Module

- GPIO Module

- CAN Module

- ADC Module

- Door Module

- Speed Module

- Switch Module

- Application Module

### Folder structure:

| Application Folder | main.c |  |
| --- | --- | --- |
|  | app.h |  |
|  | app.c |  |
| Service Folder | OS_config.h |  |
|  | OS_interface.h |  |
|  | OS.c |  |
| OnBoard Folder | DoorSensor Folder | DoorSensor_confg.h |
|  |  | DoorSensor_interface.h |
|  |  | DoorSensor.c |
|  | SpeedSensor Folder | SpeedSensor_confg.h |
|  |  | SpeedSensor_interface.h |
|  |  | SpeedSensor.c |
|  | LightSwitch Folder | LightSwitch _confg.h |
|  |  | LightSwitch _interface.h |
|  |  | LightSwitch.c |
| MCAL Folder | Timer | Timer_confg.h |
|  |  | Timer_interface.h |
|  |  | Timer.c |
|  | GPIO | GPIO_confg.h |
|  |  | GPIO_interface.h |
|  |  | GPIO.c |
|  | CAN | CAN_confg.h |
|  |  | CAN_interface.h |
|  |  | CAN.c |
|  | ADC | ADC_confg.h |
|  |  | ADC_interface.h |
|  |  | ADC.c |
| Common | STD_Types.h |  |
|  | Bit_Math.h |  |

### Typedefs and Enmus used:

| DataType | Description |
| --- | --- |
| Enum RetState_t | Used to return all function execution status |
| struct TimerConfig_st | Used to hold timer configurations |
| struct GPIOConfig_st | Used to hold GPIO configurations |
| Enum GPIO_Pin_t | GPIO pins names |
| Enum GPIO_Port_t | GPIO ports names |
| Enum GPIO_PinState_t | GPIO digital read states |
| struct CANConfig_st | Used to hold CAN configurations |
| Enum CAN_NodeID_t | CAN bus nodes available to be selected |
| Enum CAN_MsgSize_t | Data size available to be selected for CAN message |
| struct ADCConfig_st | Used to hold ADC configurations |
| Enum ADC_Channel_t | Available channels of ADC |
| Enum Doors_t | Car doors to be selected later (4 doors, 2 doors) |
| struct Door_st {Doors_t, GPIOConfig_st} | Used as object of door that holds all its data |
| Enum SpeedSensors_t | Car speed sensors to be selected |
| struct SpeedSensor_st{SpeedSensors_t, ADCConfig_st} | Used as object of speed sensors that holds all its data |
| Enum Switchs_t | Car switches to be selected |
| struct Switch_st{Switchs_t, GPIOConfig_st} | Used as object of switches that holds all its data |
| EventFlag_ECU1 | Event flag group that stores sensors values (OS feature) |

### APIs and Private functions:

| Timer Module |  |
| --- | --- |
| APIs |  |
| RetState_t Timer_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate timer peripherals with the required setting stated by user in the configuration array |
| RetState_t Timer_start(void) |  |
| Layer | MCAL |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Used to start timer counting and trigger an interrupt for each tick in the timer. |
| RetState_t callBack_register(<parameter>) |  |
| Layer | MCAL |
| Input parameters | (*void ptrToCBFunction) |
| Return value | RetState_t |
| Description | Used by upper layers to set the ISR action of timer interrupt. |

| GPIO Module |  |
| --- | --- |
| APIs |  |
| RetState_t GPIO_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate GPIO pins according to GPIO configuration array |
| RetState_t GPIO_ConfigPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIOConfig_st Config) |
| Return value | RetState_t |
| Description | Used to configure specific pin not mentioned in the configuration array |
| RetState_t GPIO_GetPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIO_Pin_t Pinx, GPIO_Port_t Portx GPIO_PinState_t* PinReading) |
| Return value | RetState_t |
| Description | Used to get specific GPIO pin |
| RetState_t GPIO_SetPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIO_Pin_t Pinx, GPIO_Port_t Portx) |
| Return value | RetState_t |
| Description | Used to set specific GPIO pin |

| CAN Module |  |
| --- | --- |
| APIs |  |
| RetState_t CAN_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate CAN peripheral according to CAN configuration array |
| RetState_t CAN_SendMessage(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (CAN_NodeID_t NodeID, uint8_t* MsgBuffer, CAN_MsgSize_t MsgSize) |
| Return value | RetState_t |
| Description | Used to send specific message to specific node on the CAN bus |

| ADC Module |  |
| --- | --- |
| APIs |  |
| RetState_t ADC_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate ADC peripheral according to ADC configuration array |
| RetState_t ADC_ConfigChannel(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (ADCConfig_st) |
| Return value | RetState_t |
| Description | Used to initiate ADC peripheral according to ADC configuration array |
| RetState_t ADC_GetAnalogRead(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (ADC_Channel_t Channel, float* SensorAnalogRead) |
| Return value | RetState_t |
| Description | Used to get the analog read of a sensor connected to a specific channel pre-defined by the user. Value is evaluated inside this API using ADC_GetDigtalRead private function to get current digital reading then map it according to the connected sensor configurations in the module. |
| Private Functions |  |
| RetState_t ADC_GetDigtalRead(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (ADC_Channel_t Channel, uint16_t* DigReading) |
| Return value | RetState_t |
| Description | Used internally in the ADC_GetAnalogRead() to evaluate the current digital value. |

| Door Module |  |
| --- | --- |
| APIs |  |
| RetState_t DoorSensor_init(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Door_st* SelectedDoorPtr) |
| Return value | RetState_t |
| Description | Used to initiate door sensor pin according to input structure stores pin used with the sensor and its configurations |
| RetState_t DoorSensor_GetRead(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Door_st* SelectedDoorPtr, uint_t* DoorSensorReading) |
| Return value | RetState_t |
| Description | Used to get specific GPIO pin |

| Speed Module |  |
| --- | --- |
| APIs |  |
| RetState_t SpeedSensor_init(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (SpeedSensor_st* SelectedSensorPtr) |
| Return value | RetState_t |
| Description | Used to initiate a speed sensor according to the sensor object (structure) configurations. |
| RetState_t SpeedSensor_GetRead(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (SpeedSensor_st* SelectedSensorPtr, float* DoorSensorReading) |
| Return value | RetState_t |
| Description | Used to get specific sensor current speed |

| Switch Module |  |
| --- | --- |
| APIs |  |
| RetState_t Switch_init(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Switch_st* Switch) |
| Return value | RetState_t |
| Description | Used to initiate a switch. |
| RetState_t Switch_GetRead(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Switch_st* Switch, uint8_t* DoorSensorReading) |
| Return value | RetState_t |
| Description | Used to get current reading of the switch button. |

| BCM Manager Module |  |
| --- | --- |
| APIs |  |
| RetState_t BCM_Manager(void) |  |
| Layer | OS Layer |
| Input parameters | (void) Utilise event flag as input |
| Return value | RetState_t |
| Description | Send Fetched sensors reading from EventFlag_ECU1 and send them over communication bus |

| Application Module |  |
| --- | --- |
| APIs |  |
| RetState_t HW_init(void) |  |
| Layer | Application Layer |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Initiate system setting according to lower layers configurations. |
| RetState_t SendSpeed(void) |  |
| Layer | Application Layer |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Send car speed every 5ms utilising timer tick |
| RetState_t SendDoorState(void) |  |
| Layer | Application Layer |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Send door state every 10ms utilising timer tick |
| RetState_t SendSwitchState(void) |  |
| Layer | Application Layer |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Send Switch state every 20ms utilising timer tick |

## Layered Architecture (ECU2)

### On Board Components:

- Buzzer (Digital)

- Lights (Digital)

### Modules:

- Timer Module

- GPIO Module

- CAN Module

- Buzzer Module

- Lights Module

- Application Module

### Folder structure:

| Application Folder | main.c |  |
| --- | --- | --- |
|  | app.h |  |
|  | app.c |  |
| Service Folder | OS_config.h |  |
|  | OS_interface.h |  |
|  | OS.c |  |
| OnBoard Folder | Buzzer Folder | Buzzer_confg.h |
|  |  | Buzzer_interface.h |
|  |  | Buzzer.c |
|  | Light Folder | Light_confg.h |
|  |  | Light_interface.h |
|  |  | Light.c |
| MCAL Folder | Timer | Timer_confg.h |
|  |  | Timer_interface.h |
|  |  | Timer.c |
|  | GPIO | GPIO_confg.h |
|  |  | GPIO_interface.h |
|  |  | GPIO.c |
|  | ADC | ADC_confg.h |
|  |  | ADC_interface.h |
|  |  | ADC.c |
| Common | STD_Types.h |  |
|  | Bit_Math.h |  |

### Typedefs and Enmus used:

| DataType | Description |
| --- | --- |
| Enum RetState_t | Used to return all function execution status |
| struct TimerConfig_st | Used to hold timer configurations |
| struct GPIOConfig_st | Used to hold GPIO configurations |
| Enum GPIO_Pin_t | GPIO pins names |
| Enum GPIO_Port_t | GPIO ports names |
| Enum GPIO_PinState_t | GPIO digital read states |
| struct CANConfig_st | Used to hold CAN configurations |
| Enum CAN_MsgSize_t | Data size available to be selected for CAN message |
| Enum Buzzer_t | Enum used to select which buzzer to operate |
| struct Buzzer_st {Buzzer_t, GPIOConfig_st} | Used as object of Buzzer that holds all its data |
| Enum Light_t | Enum used to select which Light to operate |
| struct Light_st{Light_t, GPIOConfig_st} | Used as object of Light that holds all its data |
| Enum App_Device_t | Enum used to define application used devices (Buzzer , LeftLight, Right Light) |
| EventFlag_ECU2 | Event flag group that stores sensors values (OS feature) |

### APIs and Private functions:

| Timer Module |  |
| --- | --- |
| APIs |  |
| RetState_t Timer_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate timer peripherals with the required setting stated by user in the configuration array |
| RetState_t Timer_start(void) |  |
| Layer | MCAL |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Used to start timer counting and trigger an interrupt for each tick in the timer. |
| RetState_t callBack_register(<parameter>) |  |
| Layer | MCAL |
| Input parameters | (*void ptrToCBFunction) |
| Return value | RetState_t |
| Description | Used by upper layers to set the ISR action of timer interrupt. |

| GPIO Module |  |
| --- | --- |
| APIs |  |
| RetState_t GPIO_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate GPIO pins according to GPIO configuration array |
| RetState_t GPIO_ConfigPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIOConfig_st Config) |
| Return value | RetState_t |
| Description | Used to configure specific pin not mentioned in the configuration array |
| RetState_t GPIO_GetPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIO_Pin_t Pinx, GPIO_Port_t Portx GPIO_PinState_t* PinReading) |
| Return value | RetState_t |
| Description | Used to get specific GPIO pin |
| RetState_t GPIO_SetPin(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (GPIO_Pin_t Pinx, GPIO_Port_t Portx) |
| Return value | RetState_t |
| Description | Used to set specific GPIO pin |

| CAN Module |  |
| --- | --- |
| APIs |  |
| RetState_t CAN_init(void) |  |
| Layer | MCAL |
| Input parameters | (void) Utilise configuration array of the module |
| Return value | RetState_t |
| Description | Used to initiate CAN peripheral according to CAN configuration array |
| RetState_t CAN_ReadBus(<parameters>) |  |
| Layer | MCAL |
| Input parameters | (uint8_t* MsgBuffer, CAN_MsgSize_t MsgSize) |
| Return value | RetState_t |
| Description | Used to read CAN bus (Transceiver buffer). |

| Buzzer Module |  |
| --- | --- |
| APIs |  |
| RetState_t Buzzer_init(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Buzzer_st* SelectedBuzzerPtr) |
| Return value | RetState_t |
| Description | Used to initiate Buzzer pin according to input structure. |
| RetState_t Buzzer_Set(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Buzzer_st* SelectedBuzzerPtr, GPIO_PinState_t BuzzerState) |
| Return value | RetState_t |
| Description | Used to turn buzzer on/off |

| Light Module |  |
| --- | --- |
| APIs |  |
| RetState_t Light_init(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Light_st* SelectedSensorPtr) |
| Return value | RetState_t |
| Description | Used to initiate light pins according to input structure. |
| RetState_t Light_Set(<parameters>) |  |
| Layer | OBAL |
| Input parameters | (Light_st* SelectedLightPtr, GPIO_PinState_t LightState) |
| Return value | RetState_t |
| Description | Used to turn Lights on/off |

| BCM Manager Module |  |
| --- | --- |
| APIs |  |
| RetState_t BCM_Manager(void) |  |
| Layer | OS Layer |
| Input parameters | (void) Utilise event flag as input |
| Return value | RetState_t |
| Description | Read sensors states over communication bus and store them inside EventFlag_ECU2 |

| Application Module |  |
| --- | --- |
| APIs |  |
| RetState_t HW_init(void) |  |
| Layer | Application Layer |
| Input parameters | (void) |
| Return value | RetState_t |
| Description | Initiate system setting according to lower layers configurations. |
| RetState_t OperateDigDev(<parameters>) |  |
| Layer | Application Layer |
| Input parameters | (App_Device_t SelectedDevice, GPIO_PinState_t State_t) |
| Return value | RetState_t |
| Description | Implement logic to operate devices according to system constraints. Used to operate a specific device (Light, Buzzer. Operation based on data pushed from GetSensorsValues. |

# Dynamic Design (Req. 3)

## On Board Components State Machine (ECU1)

    - Speed Sensor Component:

    - Door Sensor Component:

    - Switch Component:

## ECU State Machine (ECU1):

## ECU Sequence Diagram (ECU1):

## CPU Load (ECU1):

Assume:-

- TaskSendSpeedexe time        = 1 ms

- TaskSendDoorStateexe time= 0.5 ms

- TaskSwitchStateexe time    = 0.5 ms

- TaskBCMMangerexe time   = 1 ms

HyperPeriod = 20 ms

CPULoad= Tasks execution timeHyperPeriod=(205×1)+(2010×0.5)+(2020×0.5)+(205×1)20= 47.5%

## On Board Components State Machine (ECU2)

    - Buzzer Component:

    - Lights Component:

## ECU State Machine (ECU2):

## ECU Sequence Diagram (ECU2):

## CPU Load (ECU2):

Assume:-

- TaskBCMManagerexe time        = 1 ms

- TaskOperateDigDevexe time= 0.9 ms

HyperPeriod = 5 ms

CPULoad= Tasks execution timeHyperPeriod=(55×1)+(55×0.9)5=38%

# CAN Bus Load (Req. 3)

- CAN Bus Configurations:

Assume using CAN_LowSpeed at rate of (125 kbit/second)

Assume each sensor state is encoded in 1-bit

| Car Moving State | 0 -> Stopped | 1 -> Moving |
| --- | --- | --- |
| Door Sensor State | 0 -> Closed | 1 -> Opened |
| Light Switch Sensor State | 0 -> OFF | 1 -> ON |

So we have 3-bits of data rounded up to 1-Byte

- CAN Frame size calculations:

Calculation of bits in standard CAN frame =

1(SOF) + 11(Identifier) + 7(Control) + 8(Data) + 15(CRC) + 2 (ACK) + 7(EOF)

= 51 bit/frame

- CAN Frame Speed calculations:

125000 bit → 1 second

51          bit → x second

x = 51125000= 0.408 ms

So it takes 0.408 ms to send one frame

- CAN Bus Load calculations:

Knowing that (BCM_Manager) in ECU1 is sending one frame periodically each 5ms

framesperSecond = 1000ms / 5ms = 200 frame/sec

CAN.BusBusyTime = 200 * 0.408 ms = 81.6ms

CAN.BusLoad= 81.6/1000 = 8.16 %

Answer = 8.16 %